Voltage and Energy Utilization Enhancement           in a Stand-alone WECS Using a Fuzzy Logic Controlled SPWM Converter Interface Scheme 
A. M. Sharaf, Senior Member, IEEE

Dept. of Electrical and Computer Engineering

University of New Brunswick

Fredericton, NB, E3B 5A3 Canada

sharaf@unb.ca 

I. H. Altas, Senior Member, IEEE

Dept. of Electrical and Electronics Engineering

Karadeniz Technical University

61080 Trabzon, Turkey

ihaltas@altas.org
E. Ozkop, Student Member, IEEE

Dept. of Electrical and Electronics Engineering

Karadeniz Technical University

61080 Trabzon, Turkey

eozkop@ktu.edu.tr
Abstract--In this study, a novel sinusoidal PWM switched AC/DC/AC converter interface scheme using real novel tri-loop voltage error tracking fuzzy logic controller (FLC), to stabilize the stand alone Wind Energy Conversion Scheme (WECS) using an induction generator. The proposed novel sinusoidal Pulse Width Modulator (SPWM) switched (AC/DC/AC) converter (diode rectifier and 6-pulse IGBT inverter) interface scheme serves as a combined voltage stabilization regulator and maximum wind energy utilization and enhancement compensator.
Index Terms— Wind Energy Conversion Scheme, Fuzzy Logic Controller, AC/DC/AC Interface, PWM inverter
I. Introduction
The demand for Electrical Energy and Fossil fuel has increased continuously during the last two or three decades with energy shortage, dwindling world fossil fuel and non-renewable natural resources. This has renewed interest in green Energy and Renewable/Alternative Wind, Photovoltaic, Fuel Cell, Bio-fuels and hybrid green energy schemes to supplement, complement and even replace fossil energy consumption in specific applications such as Transportation and Drive Systems. Extensive efforts in Energy Conservation, Demand Side Management and Efficient utilization, Loss Reduction can sustain the drive for LIVE-EARTH Renewal and Sustainability-Drive. Green House Gases, Water and Air Pollution and Rising Environmental Concerns are the motivating forces behind the Search for New Sustainable Energy. The traffic congestion problems have also grown and the increasing fuel cost did increase cost of living and acted as a Catalyst for New Technology Applications in Transportation Locomotive and Drive Sectors. The general view of the international oil industry regarding world oil reserves is even more alarming. The world oil market is expected to become a seller’s market as early as 2006 and at the latest by 2016. Temporarily, the balance of power will shift towards Organization of Petroleum Exporting Countries (OPEC), but even the Middle East production is expected to start falling around 2010. A crisis in supply-demand balance is likely to emerge within 12 years as the impact of the growing demand of the developing economies competes with the high demand from developed countries for a dwindling supply [1].
In spite of the problems and concerns explained above, nowadays the majority of the world electricity is still generated by hydropower, fossil fuels and nuclear power. Instead of using conventional energy sources, which are neither renewable nor environmentally friendly besides the increasing costs, alternative energy resources such as wind, solar, geothermal, biomass and hydrogen energy should be used. The renewable energy sources share of total world energy consumption is expected to rise from 7 percent to 8 percent by 2030 [2]. 

Wind energy is widely used in modern electrical systems either as stand-alone applications or utility connected power stations. In many applications the wind energy systems are combined with other renewable types such as photovoltaic (PV) and fuel cell systems. For utility-scaled sources of wind energy, a large number of wind turbines are usually built closer to form a wind-Farm. Several electricity providers today use wind farms to supply power to their remote area customers [3].

With this increased utilization of wind energy farms as a viable renewable resource it is becoming essential to improve security and the reliability of wind power systems. For standalone wind energy schemes, serious voltage instability and loss of induction generator excitation, requiring shutdown and restarting, are usually byproducts of load excursion and severe changes as well as wind speed conditions. Conventional passive capacitor compensation device becomes ineffective when the wind energy and electric load vary randomly [4].

II. System Description 

A Standalone Wind Energy Conversion Scheme (WECS) using induction generator (IG) is studied in this paper under a time sequence of Load Excursions and Wind variations. The standalone WECS connected to the local load bus over a radial transmission line comprising the following main components, as shown in Fig. 1.
· Wind turbine
· Gear box
· Step up and step down transformers
· Distribution power lines
· Induction generator
· Stabilization interface scheme and stabilization 
· controller
· The hybrid electric load.
Each component of the proposed WECS shown in Fig. 1 is modeled in Matlab/Simulink [5] environment. The hybrid composite linear, nonlinear and motorized loads are shown in Fig. 2. 
[image: image1.png]Wind
Turbine

Self Excited
Induction
Machine

B2




Figure 1. Sample study 600 kVA wind energy conversion scheme.

[image: image2.emf]VL

R

L

L

L

Linear

Load 40%

IM

R

NL

L

NL

Induction 

Motor 30%

Nonlinear

Load 30%


Figure 2. Hybrid composite linear, nonlinear and motorized load model.  

The parameters of the proposed controller are selected by a guided trial and error off-line simulation to ensure the minimum induction load and generator voltage excursion for any large wind and load variation.
The standalone WECS sample study system unified AC system model parameters, comprising the induction generator, wind turbine, combined hybrid load, and controller parameters are all given in Appendix. The Matlab/Simulink functional model of full AC study system is given in Fig. 3. 
[image: image3.emf]WG

A

B

C

Vref (pu) 

1

Universal Bridge

A

B

C

+

-

z

1

Tri-loop Control System

Vabc_Load (pu)

Vabc_g (pu)

Vdc(pu)

Vd_ref (pu)

Vabc_inv

m

t

Terminator

PWM

GTO Inverter

g

A

B

C

+

-

Measurement system

LOAD

A

B

C

LC Filter

A

B

C

A1

B1

C1

L

Vabc_WT

Vdc

Vabc_Load

Discrete

PWM Generator

Uref

Pulses

Clock

C

B3

A

B

C

a

b

c

B2

A

B

C

a

b

c

B1

A

B

C

a

b

c

1.6kV / 600V

600 kVA

A

B

C

a

b

c


Figure 3.  The Matlab/Simulink functional model of full AC study system.

The sample WECS standalone scheme was subjected to severe combined sequence of load switching/load variation/load excursion and wind speed variation and gusting. The novel tri-loop dynamic voltage tracking and FLC control scheme is shown in Fig. 4. The control signal adjusts the sinusoidal pulse width modulated IGBT six pulse inverter. The system real time dynamic response for a combined load/wind excursion time sequence as follows:

t=0.03s Linear Load excursion applied, +30%

t=0.04s Linear Load excursion removed, +30%

t=0.05s Linear Load excursion applied, -30%

t=0.06s Linear Load excursion removed, -30%

t=0.07s Wind Speed excursion applied, -30%

t=0.08s Wind Speed excursion removed, -30%

t=0.09s Wind Speed excursion applied, +30%

t=0.10s Wind Speed excursion removed, +30%

The details of the fuzzy logic control (FLC) blocks shown in Fig. 4 are discussed in [6, 7] and they are not going to be repeated here.  
[image: image4.emf]m

2

Vabc_inv

1

gama_load

-

K-

gama_generator

-

K-

gama_dc

-

K-

dq0_to_abc

Transformation

dq0

sin_cos

abc

abc_to_dq0

Transformation 1

abc

sin_cos

dq0

abc_to_dq0

Transformation

abc

sin_cos

dq0

Vq_ref (pu)3

1

Vq_ref (pu)2

0

Vq_ref (pu)1

0

Vq_ref (pu)

0

V0

0

Selector

Vd Vq3

Selector

Vd Vq2

Selector

Vd Vq1

Selector

Vd Vq

hypot

Fuzzy Logic Controller2

REFERENCE

FEEDBACK

u(k)

Fuzzy Logic Controller1

REFERENCE

FEEDBACK

u(k)

Discrete

Virtual PLL

60 Hz

Freq

Sin_Cos

wt

Vd_ref (pu)

4

Vdc(pu)

3

Vabc_g (pu)

2

Vabc_Load (pu)

1

modulation index


Figure 4. Dynamic tri-loop global voltage error tracking FL controlled scheme for voltage stabilization at generator, load and inverter buses.

III. Simulation Results
The WECS dynamic performance is compared for the two cases, with and without the AC/DC/AC PWM converter based on the novel tri-loop dynamic voltage tracking tri-loop FLC. Fig. 7 shows the system dynamic response including generator voltage (RMS), load voltage (RMS), DC-link voltage, generator current (RMS), average generator power, the voltage-current 2-dimensional phase portraits for the wind energy system controller interface system as shown in Fig. 7.. 

[image: image5.emf]0 0.02 0.04 0.06 0.08 0.1

0

0.2

0.4

0.6

0.8

1

time (s)

Vg__rms (pu)

                       [image: image6.emf]0 0.02 0.04 0.06 0.08 0.1

0

0.2

0.4

0.6

0.8

1

time (s)

Vg__rms (pu)


	(a) RMS value of the generator voltage without SPWM converter.
	(b) RMS value of the generator voltage with the controlled SPWM converter.


         [image: image7.emf]0 0.02 0.04 0.06 0.08 0.1

0

0.5

1

1.5

2

time (s)

Ig__rms (pu)

                       [image: image8.emf]0 0.02 0.04 0.06 0.08 0.1

0

0.2

0.4

0.6

0.8

1

1.2

1.4

time (s)

Ig__rms (pu)


	(c) RMS value of the generator current without SPWM converter.
	(d) RMS value of the generator current with the controlled SPWM converter.


[image: image9.emf]0 0.02 0.04 0.06 0.08 0.1

0

0.2

0.4

0.6

0.8

1

time (s)

VLoad__rms (pu)

                    [image: image10.emf]0 0.02 0.04 0.06 0.08 0.1

0

0.02

0.04

0.06

0.08

0.1

0.12

0.14

0.16

time (s)

VLoad__rms (pu)


	(e) RMS value of the load voltage without SPWM converter.
	(f) RMS value of the load voltage with the controlled SPWM converter.


                        [image: image11.emf]0 0.02 0.04 0.06 0.08 0.1

-0.5

0

0.5

1

1.5

2

2.5

3

time (s)

Pg (pu)

                       [image: image12.emf]0 0.02 0.04 0.06 0.08 0.1

-1

-0.5

0

0.5

1

1.5

2

2.5

3

3.5

time (s)

Pg (pu)


	(g) Active power generated by the WECS without SPWM converter.
	(h) Active power generated by the WECS with the controlled SPWM converter.


   [image: image13.emf]0 0.02 0.04 0.06 0.08 0.1

0

0.5

1

1.5

2

2.5

time (s)

Pload (pu)

                         [image: image14.emf]0 0.02 0.04 0.06 0.08 0.1

-0.1

-0.05

0

0.05

0.1

0.15

0.2

0.25

0.3

time (s)

PLoad (pu)


	(i) Active power dissipated by the load without SPWM converter.
	(j) Active power dissipated by the load with the controlled SPWM converter.


                    [image: image15.emf]0 0.1 0.2 0.3 0.4 0.5 0.6 0.7

0

0.5

1

1.5

2

Vg__rms (pu)

Ig__rms (pu)

                   [image: image16.emf]0 0.2 0.4 0.6 0.8 1

0

0.2

0.4

0.6

0.8

1

1.2

1.4

Vg__rms (pu)

Ig__rms (pu)


	(k) RMS current-voltage trajectory at generator terminals without
 SPWM converter.
	(l) RMS current-voltage trajectory at generator terminals with SPWM converter.


Figure 5. WECS performance without and with the SPWM converter for a combined excursion sequence.

V. Conclusion
The paper presents a novel low cost dynamic voltage error tri-loop fuzzy logic controller and sinusoidal PWM driven IGBT inverter scheme for stand alone wind energy system. This PWM power converter scheme is extremely effective in ensuring voltage stabilization and enhancing power/energy utilization under severe load and wind prime mover/wind velocity excursion. A tri-loop dynamic error driven Fuzzy Logic Controller is designed and employed in this application. The use of tri-loop dynamic error signal to the controller ensures that all changes, excursions or variations in WECS, in Loads, and in DC interface system are included in the controller. 
VI. Appendıx
A.1 Simple Wind Turbine Model (Quasi-static model)

[image: image17.wmf]3

23

11

22

W

WPWPW

WW

V

TARCVACVk

rr

lww

===


Where

[image: image18.wmf]r

is the specified density of air (1.25kg/m2)

[image: image19.wmf]A

is the area swept by the blades

[image: image20.wmf]R

is the radius of the rotor blades

[image: image21.wmf]P

C

is power conversion coefficient


[image: image22.wmf]l

is the tip speed ratio


[image: image23.wmf]W

w

is the wind turbine velocity in rpm

[image: image24.wmf]k

is equivalent coefficient in per unit (0.745)

A.2 Induction Generator 
3 phase, 2 pairs of poles, Vg=1.6 kV (L-L), Sg=600 KVA, Cself=150uF/Phase

[image: image25.wmf]''

0.016,0.06,0.015,0.06

slsrlr

RLRL

====



[image: image26.wmf]3.5,2,0,2

m

LHFp

====


A.3 Combined Hybrid AC Load model (@ V=1.0 pu)

A.3.1 Linear PQ Load (40%) 


[image: image27.wmf]0.4,0.4

LL

PpuQpu

==


A.3.2 Nonlinear (Voltage-dependent type) PQ Load (30%)


[image: image28.wmf](),()

gg

oo

gogo

VV

PPQQ

VV

aa

==



[image: image29.wmf]0.3,0.3,1.0,

23,23()

oogo

PpuQpuVpu

Nonlinearityorder

ab

===

=-=-


A.3.3 Three phase squirrel cage induction motor inrush type PQ load (30%)
Power: 
[image: image30.wmf]0.3

M

Spu

=


Stator resistance and leakage inductance: 
[image: image31.wmf]0.0201,0.0349

sls

RpuLpu

==


Rotor resistance and leakage inductance:


[image: image32.wmf]0.0377,0.0349

rlr

RpuLpu

==


Magnetizing inductance: 
[image: image33.wmf]1.2082

m

Lpu

=


Pole pairs: 2

A.4 Per unit base values used

Sbase=600KVA, Vbase=1.6kV (L-L)

A.5 DC-link RLC filter parameters

[image: image34.wmf]0.1,0.2,5000

fff

RLmHCuF

=W==


A.6 Weight factors

[image: image35.wmf]10,10,0.1

vgvdcvload

ggg

===


A.7. SPWM switching frequency

[image: image36.wmf]2,6

sw

fkHzpulseIGBToperation

=


References
[1] International Energy Outlook 2007, Energy Information Administration (EIA), http://www.eia.doe.gov/iea.

[2] E. Ozkop, “Sensorless BLDC control of permanent magnet axial flux motor,” MSc dissertation, Gent University, Belgium, July 2006.

[3] A.M. Sharaf, A. Aljankawey and I.H. Altas, “A Novel voltage stabilization control scheme for standalone wind energy Conversion systems,” International Conference on Clean Electrical Power, ICCEP’07, May 21-23, 2007, Capri, Italy.
[4] A.M. Sharaf and W. Wang, “A Low-cost voltage stabilization and power quality enhancement scheme for a small renewable wind energy scheme,” 2006 IEEE International Symposium on Industrial Electronics, Quebec, Canada.
[5] The Mathworks, Inc, as of September 25, 2006: http://www.matworks.com
[6] I.H. Altas and A.M. Sharaf, “Novel Maximum Power Fuzzy Logic Controller for Photovoltaic Solar Energy Systems,” Renewable Energy (2007), doi:10.1016/j.renene.2007.03.002.
[7] I.H. Altas and A.M. Sharaf, “A Generalized Direct Approach for Designing Fuzzy Logic Controllers in Matlab/Simulink GUI Environment,” International Journal of Information Technology and Intelligent Computing, Int. J. IT&IC no.4 vol.1, 2007.
_1256458077.unknown

_1256458395.unknown

_1256458729.unknown

_1256458778.unknown

_1256459217.unknown

_1256459310.unknown

_1256458909.unknown

_1256458758.unknown

_1256458576.unknown

_1256458123.unknown

_1256458130.unknown

_1256458078.unknown

_1256457451.unknown

_1256457460.unknown

_1256457488.unknown

_1256457452.unknown

_1256457313.unknown

_1256457363.unknown

_1256457440.unknown

_1256457259.unknown

